Operation Manual of Trim-off SK6368 PDT

Hardware:

Computer(x86) x1,

USB3.0 cable with micro B plug x1,

Skymedi Switch PCBA (SATA to USB3.0) x1,

Generic MSATA change to SATA PCBA x1,

Software:

Operation system: Windows xp sp3 (necessary condition)

Skmedi PDT program(V4629 trim off)

Operation steps:

- Plug USB3.0 cable with two switch PCBAs into computer(x86) (compatible with USB2.0), then boot the computer.

- Program path:  C:\v4629_TrimOff_20140505\tools\SKPdt.exe (Program must be placed in root of C: because of path setting) .

- Turn on the switch on the mSATA to USB PCBA, then open this program.

- Press button “Setting”, you can change model no and serial no as below picture. After completing the setting, you must press the button “Save and Exit”.

- Press the button “Start”, you can see the process like below. If appear 100%, mSATA SSD has been successfully finished. After turning off the switch, you can unplug the switch PCBA(MSATA to SATA), change mSATA SSD, then repeat the same steps.

- You can use CrystalDiskInfo to check its Trim function and health status.